Grade 9 IT — FreeCAD Phone Stand

Auto-Gradable Rubric (100 points)

Subject: Information Technology

Grade level: Grade 9 (High school / intermediate)

Assignment: Design and model a phone stand in FreeCAD

Submission format: Single .FCStd file per student

## Section 1: Technical Execution (45 points)

| # | Criterion | Points |
| --- | --- | --- |
| 1.1 | File submitted correctly (.FCStd, opens without error) | 5 |
| 1.2 | PartDesign Body container present | 5 |
| 1.3 | At least one sketch present | 5 |
| 1.4 | Sketch is fully constrained | 12 |
| 1.5 | 3D feature created (Pad / Pocket / Revolve) | 13 |
| 1.6 | File saves cleanly (no broken references) | 5 |

## Section 2: Parametric Modelling Technique (30 points)

| # | Criterion | Points |
| --- | --- | --- |
| 2.1 | Used sketch-then-feature workflow | 10 |
| 2.2 | Used appropriate constraint types (geometric + dimensional) | 10 |
| 2.3 | Used at least 2 different feature types | 5 |
| 2.4 | Objects renamed meaningfully | 5 |

## Section 3: Design Requirements (25 points)

| # | Criterion | Points |
| --- | --- | --- |
| 3.1 | Overall dimensions within spec (≤ 150 × 100 × 100 mm) | 6 |
| 3.2 | Phone slot / groove of appropriate width (8–15 mm) | 6 |
| 3.3 | Stable base (footprint width ≥ 60 mm) | 7 |
| 3.4 | Wall thickness sufficient for 3D printing (≥ 2 mm) | 6 |

Total: 100 points

## Grade Bands

| Percentage | Grade band |
| --- | --- |
| 85 – 100 | Excellent |
| 70 – 84 | Good |
| 55 – 69 | Satisfactory |
| 40 – 54 | Developing |
| Below 40 | Needs resubmission |

## Notes on the Rubric

Why no creativity / aesthetic criteria?

This rubric is fully auto-gradable. Aesthetic and creative aspects of the design are graded separately (or in a follow-up oral / written reflection), so that the script can produce consistent, objective marks across the whole class.

Why heavy weighting on sketch constraint and 3D feature?

These two criteria together (25 points) test whether the student understands the core idea of parametric CAD: a sketch is the basis for a feature, and a fully constrained sketch produces predictable geometry. A student who skips either has misunderstood the central concept.

Why include object renaming for only 5 points?

It's a habit-building criterion. In real-world CAD work, leaving everything as Sketch001 and Pad003 makes a file unreadable to anyone else. Five points is enough to nudge students towards good practice without dominating the mark.